Practical 1: LAN Design with Subnetting and Router ConfigurationAim:

Design a Local Area Network (LAN) using 3 PCs connected to a router.

Instructions:

• Assign each PC a unique subnet.

• Configure IP addresses and subnet masks.

• Configure the router interfaces to connect the subnets.

• Ensure all devices can communicate via the router.

#### 1. Add Devices

- Drag and drop the following from the bottom device list:

  - 1 Router (Router0)

  - 2 PCs (PC0 and PC1)

#### 2. Connect Devices

- Use Copper Cross-Over Cable:

  - Connect PC0 → Router0

  - Connect PC1 → Router0

You can use FastEthernet0 on PCs and GigabitEthernet0/0 or 0/1 on the router depending on availability.

#### 3. Configure IP Addresses

- PC0 Configuration:

  - Go to Desktop > IP Configuration

  - IP Address: 192.168.1.1

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.1.254

- PC1 Configuration:

  - IP Address: 192.168.2.1

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.2.254

#### 4. Configure Router Interfaces

- Click on Router0 > CLI or Config tab, then configure interfaces:

enable

configure terminal

interface gigabitEthernet0/0

ip address 192.168.1.254 255.255.255.0

no shutdown

exit

interface gigabitEthernet0/1

ip address 192.168.2.254 255.255.255.0

no shutdown

exit

#### 5. Verify Connection

- Use Simulation Mode to test ICMP (ping):

  - From PC0 Desktop > Command Prompt, type:

  - ping 192.168.2.1

You should see the ping packets go from PC0 → Router → PC1 and a successful reply if configured correctly.

Practical 2: Router-to-Switch Connection and Basic Configuration

Aim:

Connect a router to a switch and perform initial router configurations.

Instructions:

• Connect the router to the switch using proper cabling.

• Assign IP addresses to connected devices.

• Configure basic settings such as hostname, interface IP, and default gateway.

• Verify end-to-end connectivity using ping.

### 1. Add Devices

- 1 Router (e.g. Router0)

- 1 Switch (e.g. Switch0)

- 2 PCs (e.g. PC0, PC1)

### 2. Connect Devices

- Use Copper Straight-Through Cable:

  - Router0 ↔ Switch0 (e.g., Gig0/0 to FastEthernet0/1)

  - PC0 ↔ Switch0 (e.g., FastEthernet0/2)

  - PC1 ↔ Switch0 (e.g., FastEthernet0/3)

### 3. Configure VLANs on the Switch

- Go to the Switch CLI:

enable

configure terminal

vlan 10

name VLAN10

vlan 20

name VLAN20

exit

- Assign switch ports to VLANs:

interface fastEthernet0/2

switchport mode access

switchport access vlan 10

exit

interface fastEthernet0/3

switchport mode access

switchport access vlan 20

exit

- Configure the trunk port (connection to router):

interface fastEthernet0/1

switchport mode trunk

exit

### 4. Configure Router Subinterfaces for Inter-VLAN Routing

Go to the Router CLI:

enable

configure terminal

interface gigabitEthernet0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

exit

interface gigabitEthernet0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

exit

interface gigabitEthernet0/0

no shutdown

exit

### 5. Configure IP on PCs

#### PC0:

- IP Address: 192.168.10.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.10.1

#### PC1:

- IP Address: 192.168.20.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.20.1

### 6. Test Connectivity

- Go to PC0 > Command Prompt:

ping 192.168.20.2

You should receive successful replies — meaning inter-VLAN routing is working properly.

Would you like to add DHCP, DNS, or more VLANs to this setup?

Practical 3: Setup a Small Office/Home Office (SOHO) Network

Aim:

Design and configure a basic SOHO network with internet access.

Instructions:

• Use a router, switch, and multiple PCs.

• Assign appropriate IP addresses.

• Configure DHCP (if required).

• Simulate internet connectivity using a cloud device.

### 1. Add Devices

From the device selection at the bottom of Cisco Packet Tracer:

- 1 Switch (e.g., Switch0)

- 5 PCs (e.g., PC0 to PC4)

- 1 Server (e.g., Server0)

### 2. Connect Devices to the Switch

Use Copper Straight-Through Cable to connect each device to the switch:

- PC0 to Switch0

- PC1 to Switch0

- PC2 to Switch0

- PC3 to Switch0

- PC4 to Switch0

- Server0 to Switch0

(Connect all to different FastEthernet ports, like Fa0/1 to Fa0/6)

### 3. Assign IP Addresses

Use manual IP configuration:

| Device | IP Address | Subnet Mask | Default Gateway (optional) |
| --- | --- | --- | --- |
| PC0 | 192.168.1.1 | 255.255.255.0 | 192.168.1.254 (if needed) |
| PC1 | 192.168.1.2 | 255.255.255.0 | 192.168.1.254 |
| PC2 | 192.168.1.3 | 255.255.255.0 | 192.168.1.254 |
| PC3 | 192.168.1.4 | 255.255.255.0 | 192.168.1.254 |
| PC4 | 192.168.1.5 | 255.255.255.0 | 192.168.1.254 |
| Server0 | 192.168.1.10 | 255.255.255.0 | 192.168.1.254 |

To assign IPs:

- Click each PC or Server

- Go to Desktop > IP Configuration

- Manually enter the IP Address and Subnet Mask

### 4. Test Connectivity

To verify the LAN is working:

- Open Command Prompt on PC0

- Type:

- ping 192.168.1.3

- ping 192.168.1.10

If responses return without packet loss, your LAN setup is working correctly.

- Click on the PC (e.g., PC1).

- Go to the Desktop tab.

- Click on IP Configuration.

- Select Static.

- Enter:

  - IP Address: 10.0.0.X (e.g., 10.0.0.4)

  - Subnet Mask: auto-fills as 255.0.0.0

  - Default Gateway: 10.0.0.1

- Close the window.

Practical 4: Secure a Cisco Router or Switch with Passwords

Aim:

Secure access to Cisco devices using different types of passwords.

Instructions:

• Set a console password for accessing the device locally.

• Set an enable password for privileged mode.

• Configure VTY (telnet/SSH) passwords for remote access.

• Save the configuration and test login access

#### 1. Start the CLI

When prompted, press Enter.

#### 2. Enter Privileged EXEC Mode:

enable

#### 3. Enter Global Configuration Mode:

configure terminal

#### 4. Set a Hostname (Optional):

hostname Switch0

#### 5. Set Console Password:

line console 0

password cisco

login

exit

#### 6. Set Enable Password:

enable password class

#### 7. Set IP Address for VLAN 1 (Switch Management):

interface vlan 1

ip address 192.168.1.2 255.255.255.0

no shutdown

exit

#### 8. Set Default Gateway (for remote access):

ip default-gateway 192.168.1.1

#### 9. Save the Configuration:

exit

write memory

#### 1. Enter Privileged EXEC Mode:

enable

#### 2. Enter Global Configuration Mode:

configure terminal

#### 3. Set Console Password:

line console 0

password cisco

login

exit

#### 4. Set Enable Password:

enable password class

#### 5. Save the Configuration:

write memory

✅ Now your router is secured with:

- Console access requiring the password cisco

- Privileged mode access requiring the password class

Practical 5: Configure RIP v2 on Multiple Router

## 🔁 Static Routing Between 3 Routers

### 🛠 Step 1: Assign IP Addresses to Router Interfaces

Do this for each router:

enable

configure terminal

interface <INTERFACE_NAME>

ip address <IP_ADDRESS> <SUBNET_MASK>

no shutdown

exit

➡️ Example from your image:

Router1:

interface g0/0

ip address 12.0.0.1 255.255.255.0

no shutdown

exit

interface g0/1

ip address 13.0.0.1 255.255.255.0

no shutdown

Router2:

interface g0/0

ip address 12.0.0.2 255.255.255.0

no shutdown

exit

interface g0/1

ip address 23.0.0.2 255.255.255.0

no shutdown

Router3:

interface g0/0

ip address 13.0.0.3 255.255.255.0

no shutdown

exit

interface g0/1

ip address 23.0.0.3 255.255.255.0

no shutdown

### 🚏 Step 2: Configure Static Routes

On each router, add static routes to the networks not directly connected.

#### Router1:

ip route 23.0.0.0 255.255.255.0 12.0.0.2

#### Router2:

ip route 13.0.0.0 255.255.255.0 12.0.0.1

#### Router3:

ip route 12.0.0.0 255.255.255.0 13.0.0.1

### ✅ Step 3: Verify Connectivity

Use the ping command from one router to another:

ping 13.0.0.3

ping 12.0.0.1

ping 23.0.0.2

Or use the Simulation Mode like you've shown — all pings should be successful.

Practical 6: VLAN and VTP Configuration on Switches

## ✅ Static Routing in Cisco Packet Tracer (3 Routers)

### 🖥 Step 1: Connect the Routers

Use copper cross-over cables to connect the routers like this:

Router1 ↔ Router2 ↔ Router3 ↔ Router1

(Connect via GigabitEthernet or FastEthernet ports)

### ⚙️ Step 2: Assign IP Addresses

Go to each router CLI and enter:

#### Router1

enable

configure terminal

interface g0/0

ip address 12.0.0.1 255.255.255.0

no shutdown

exit

interface g0/1

ip address 13.0.0.1 255.255.255.0

no shutdown

exit

#### Router2

enable

configure terminal

interface g0/0

ip address 12.0.0.2 255.255.255.0

no shutdown

exit

interface g0/1

ip address 23.0.0.2 255.255.255.0

no shutdown

exit

#### Router3

enable

configure terminal

interface g0/0

ip address 13.0.0.3 255.255.255.0

no shutdown

exit

interface g0/1

ip address 23.0.0.3 255.255.255.0

no shutdown

exit

### 📍 Step 3: Add Static Routes

#### Router1

ip route 23.0.0.0 255.255.255.0 12.0.0.2

#### Router2

ip route 13.0.0.0 255.255.255.0 12.0.0.1

#### Router3

ip route 12.0.0.0 255.255.255.0 13.0.0.1

### 🔄 Step 4: Test the Network

- Click on Simulation Mode.

- Use Add Simple PDU (the lightning icon).

- Click Router1, then Router3 (and vice versa).

- Check that the packets show "Successful" in the event list (as in your screenshot).